# Lab 1: System Modeling in Maple

We are going to do some virtual experiments by simulating a parallel RLC circuit using Maple. This system is shown in the figure below.

Figure 1: A Parallel RLC Circuit

We will choose a capacitance c = 1 f so the transfer function model is

X(s)F(s)=1s2+cs+k

- Manually derive the general transfer function for the system by hand.

- Compare the effects of a unit response and unit impulse function to disturb the system:

  - Using c = 1, k = 2, and c = 3 derive the time domain equations for both inputs and plot your results. Comment on the results.

  - Using c = 1, k = 2, and c = 2 derive the time domain equations for both and plot your results. Comment on the results.

- Compare and contrast on the system response to the two inputs. What effect does the damping coefficient play on the system? What effect would a damping coefficient of zero have on the system?

# Lab 2: System Modeling in MapleSim

We are going to do some virtual experiments by simulating a mass-spring-damper system using Maple and MapleSim. This system is shown in the figure below and we assume that the friction between the surface and mass can be ignored.

Figure 1: A Parallel RLC Circuit

We will choose a mass m=1kg so the transfer function model is

X(s)F(s)=1s2+cs+k

- Manually derive the general transfer function for the system by hand.

- Construct the model in MapleSim and extract the equations for the system. Use a step signal with a height of 1 for the force input. Compare the results to your hand calculations.

- Clearly, the original transfer function model is parameterized by c and k. Can you convert it into a transfer function model that is parameterized by ζ (damping ratio) and ωn (natural frequency)? Analyze the relationship between c, k and ζ, ωn. What is your opinion on how c and k affect the damping and natural frequency?

- Suppose that we have the following spring and damping values:

| k = 1, 4, 9, 16, 25 Nm | c = 0.5, 1.0, 1.5, 2.0, 2.5 N sm |
| --- | --- |

- Using a spring constant of k = 1 obtain the results of a 10 second simulation of a unit-step response x(t) to f(t) for dampers with c = 0.5, 1.0, 1.5, 2.0, 2.5 N sm, respectively.

- Using a damping ratio of c =1 obtain the results of a 10 second simulation of a unit-step response x(t) to f(t) for springs with k = 1, 4, 9, 16, 25 Nm, respectively.

- Analyze the plots obtained in step 2.

- If you are required to improve the system’s damping ratio or natural frequency using springs and dampers, discuss possible solutions.

# Lab 3: PID Control in Maple and MapleSim

We are going to do some virtual experiments by simulating a mass-spring-damper system using Maple and MapleSim. This system is shown in the figure below and we assume that the friction between the surface and mass can be ignored.

Figure 1: A Parallel RLC Circuit

We will choose a mass m=1kg so the transfer function model is

X(s)F(s)=1s2+cs+k

- Manually derive the general transfer function for the system by hand.

- Construct the model in MapleSim and extract the equations for the system. Use a step signal with a height of 1 for the force input. Compare the results to your hand calculations.

- Clearly, the original transfer function model is parameterized by c and k. Can you convert it into a transfer function model that is parameterized by ζ (damping ratio) and ωn (natural frequency)? Analyze the relationship between c, k and ζ, ωn. What is your opinion on how c and k affect the damping and natural frequency?

- Suppose that we have the following spring and damping values:

| k = 1, 4, 9, 16, 25 Nm | c = 0.5, 1.0, 1.5, 2.0, 2.5 N sm |
| --- | --- |

- Using a spring constant of k = 1 obtain the results of a 10 second simulation of a unit-step response x(t) to f(t) for dampers with c = 0.5, 1.0, 1.5, 2.0, 2.5 N sm, respectively.

- Using a damping ratio of c =1 obtain the results of a 10 second simulation of a unit-step response x(t) to f(t) for springs with k = 1, 4, 9, 16, 25 Nm, respectively.

- Analyze the plots obtained in step 2.

- If you are required to improve the system’s damping ratio or natural frequency using springs and dampers, discuss possible solutions.