# Lab 1: System Modeling in Maple

We are going to do some virtual experiments by simulating a mass-spring-damper system using Maple. This system is shown in Figure 1 below:

f(t)

Figure 1: A Mass-Spring-Damper System

We will use a mass m=1 kg so the transfer function model is

X(s)F(s)=1s2+cs+k

- Manually derive the general transfer function for the system by hand.

- Compare the effects of a unit response and unit impulse function to disturb the system:

  - Using m = 1, k = 2, and c = 3 derive the time domain equations for both inputs and plot your results. Comment on the results.

  - Using m = 1, k = 2, and c = 2 derive the time domain equations for both and plot your results. Comment on the results.

- Compare and contrast on the system response to the two inputs. What effect does the damping coefficient play on the system? What effect would a damping coefficient of zero have on the system?

# Lab 2: System Modeling in MapleSim

We are going to do some virtual experiments by simulating a mass-spring-damper system using Maple and MapleSim. This system is shown in Figure 1 below:

f(t)

Figure 1: A Mass-Spring-Damper System

We will use a mass m=1 kg so the transfer function model is

X(s)F(s)=1s2+cs+k

- Manually derive the general transfer function for the system by hand.

- Construct the model in MapleSim and extract the equations for the system. Use a step signal with a height of 1 for the force input. Compare the results to your hand calculations.

- Clearly, the original transfer function model is parameterized by c and k. Can you convert it into a transfer function model that is parameterized by ζ (damping ratio) and ωn (natural frequency)? Analyze the relationship between c, k and ζ, ωn. What is your opinion on how c and k affect the damping and natural frequency?

- Suppose that we have the following spring and damping values:

| k = 1, 4, 9, 16, 25 Nm | c = 0.5, 1.0, 1.5, 2.0, 2.5 N sm |
| --- | --- |

- Using a spring constant of k = 1 obtain the results of a 10 second simulation of a unit-step response x(t) to f(t) for dampers with c = 0.5, 1.0, 1.5, 2.0, 2.5 N sm, respectively.

- Using a damping ratio of c =1 obtain the results of a 10 second simulation of a unit-step response x(t) to f(t) for springs with k = 1, 4, 9, 16, 25 Nm, respectively.

- Analyze the plots obtained in step 2.

- If you are required to improve the system’s damping ratio or natural frequency using springs and dampers, discuss possible solutions.

# Lab 3: Control design in Maple and MapleSim

We are going to do some virtual experiments by simulating a mass-spring-damper system using Maple and MapleSim. This system is shown in the figure:

f(t)

Figure 1: A Mass-Spring-Damper System

For our model, we will use m=1 kg, c=3 N sm, and k=2Nm so the transfer function model is

X(s)F(s)=1s2+3s+2

Using the provided MapleSim model, develop a P, PI, and PID controller to dampen out the effect of a sinusoidal disturbance applied to the mass.

- Open the MapleSim model and load the attachment Project > Attachments > Documents > ControlDesing.mw

- In Maple, select the subsystem msd2 and press “Load Selected Subsystem.” The model information will be loaded into the template.

- Press “Toggle Symbolic” to keep the equations in their symbolic form. Press “Reassign Equations” to update the template.

- Execute the commands sys := DiffEquation(DAEs, F(t), x(t)) and tf := TransferFunction(sys). These commands take the differential equation of the system and convert it to transfer function.

- Select P from the controller type dropdown. The closed loop transfer function is generated.

- Execute the commands TFClosedloopParam := eval(TFclosedloop, {c = 3, kspr = 2, m = 1}) and response := 1/s*TFClosedloopParam. These commands assign the desired parameters to the equation and apply a unit step to the input.

- Copy and paste the transfer function created above into the window.

- Move the sliders for the controller gains and explore the effects on the system response. How do the gains effect the system?

- When you have tuned your system accordingly, export your gains back to the model by pressing “Return to MapleSim.”

- Return to the MapleSim environment and run your simulation and compare the controlled response to the non-controlled model.

- Repeat steps 4-9 for PI and PID controllers.